# PLANS REVIEW CHECK LIST - PUBLIC SPA POOL - DOH/BOAF version May 3, 2012

Date_____________

County_____________________________________ Date Received__________________ Log#_____________

Project_____________________________________________________________________________________

Engineer_______________________________________________ Original____ Revision____ Modification____

Reviewed By_____________________________________ SP-_______________________File #____________

Date Re-Submittal Received:___________________________ Date of 2nd Review:_______________________

_________________________________________________________________________________________________

Items needing correction or clarification are marked by an "X" beside the appropriate section number of the Florida Administrative Code citation (2009 64E-9, FAC, or 2010 FBC 424.1).  We have left the 64E-9 requirements in this checklist because they are critical for public health and therefore the pool will be checked for these items by the County Health Department at the first operating permit inspection after the Building Official’s approval of the construction.

Reviewer:  Note that section 424.1.8.1 requires that “Spa pools shall meet the requirement of Sections 424.1.1 through 424.1.6.5, unless specifically indicated otherwise.”

Use the Pool Checklist for the sections 424.1.1 (Flood Hazard Areas) through 424.1.6.5 (Recirculation and Treatment Systems) in addition to this checklist for IWFs.  From: www.myfloridaeh.com/water/swim/index.html

Florida Building Code or Florida Administrative Code

Section

424.1.8.2	___Color, pattern or finish of the interior does not obscure existence of objects or surfaces.

424.1.8.3	___Minimum water depth is 2-1/2 feet and maximum water depth is 4 feet

(except swim spas may have a maximum 5-foot depth).

424.1.8.3	___Spas over 200 square feet of water surface area have depth markers.

424.1.8.4	___Steps or ladders are provided (one set for each 75 feet, or major fraction thereof,

of pool perimeter).

424.1.8.4	___Steps for spas with more than 200 square feet of water surface area comply with

Section 424.1.2.5 for pools.

424.1.8.4	___Steps for spas with less than 200 square feet of water surface shall comply with the following:

1.  Step treads have a minimum width of 10 inches for a minimum continuous tread length of 12 inches.

2.  Step riser heights do not exceed 12 inches.

424.1.8.4	___Intermediate treads and risers between the top and bottom treads and risers are uniform

in height and width.

424.1.8.4	___Steps have contrasting markings on the leading edges of the intersections of the treads and risers.

424.1.8.4	___Submerged benches have contrasting markings on the leading edges.

424.1.8.4.1	___Handrails are provided for all steps and are anchored in the bottom step and in the deck.

424.1.8.4.1	___Handrails are located to provide maximum access to the steps, and extend

at least 28 inches above the deck.

424.1.8.4.2	___Where figure 4 handrails are used, they are anchored in the deck, and extend laterally

to any point above the bottom step.

424.1.8.4.2	___Figure 4 handrails are located to provide maximum access to the steps, and extend

at least 28 inches above the deck.

424.1.8.5	___Spa decks have a minimum 4 foot wide unobstructed width around the entire pool perimeter

except: spa pools less than 120’ sq. of water surface may have a min. 4’ wide unobstructed deck

around a min. of 50% of the pool perimeter.  If 120’ sq. or greater, only 10% may by obstructed.

424.1.8.5	___Decks less than 4 feet wide have barriers to prevent their use.

424.1.8.5	___Deck is not more than 10 inches below the top of the pool.

424.1.8.6.1	___Return lines of spa therapy or jet systems are independent of recirculation filtration and heating systems.

424.1.8.6.2	___Therapy or jet pumps take suction from a collector tank (collector tank size to take this

flow gallonage into account).

424.1.8.7	___Spa pools with less than 20 feet of perimeter have a minimum of 2 equally spaced adjustable inlets.

424.1.8.8	___Spa pool has a minimum filtration turnover of one every thirty minutes.

424.1.8.8	___Piping, fittings and hydraulic requirements comply with Section 424.1.6.5

424.1.8.8	___All recirculation lines to and from the pool are individually valved with proportional type control valves.

424.1.8.9	___Spa pool with more than 200 square feet of water surface area has provision for vacuuming.

64E-9.008(7)	___General pool rules sign required to be posted.

64E-9.010(6)(c)	___Cold plunge spa has chiller.  (therapy/jet system not required)

64E-9.010(6)(d)	___Heated system provides 15 min. patron activated timer on therapy pump circuit.

64E-9.010(13)	___Automated Oxidation Reduction Potential (ORP) & pH controllers are provided.

64E-9.010(14)(a)	___Maximum water temperature 104°F is posted on pool rules sign.

64E-9.010(14)(b,c)  ___Child supervision and health advisories are posted on pool rules sign.

64E-9.010(14)(d)	___Maximum use 15 minutes is posted on pool rules sign.

64E-9.010(15)	___A clock is visible from the spa.

64E-9.010(16)	___If spa has emergency cut-off switch, 80 decibel alarm & signage is provided.

OFFSET SPA

424.1.8.10	___If the spa pool is part of a conventional pool, it is offset from the main pool area,

with the same water depth as the main pool area.

424.1.8.10	___The offset spa pool meets all other requirements of this section.

424.1.8.10	___The deck area at the offset spa is protected by 30 inch high connected stanchions or other impediment.

424.1.8.10	___The deck perimeter of the offset spa area does not exceed 15% of the total swimming pool perimeter.

424.1.8.10	___All benches have contrasting markings on the leading edges of the intersection of the bench seats.

424.1.8.10	___Tile used for intersecting edge markings is slip resistant.

Revised for 2009 64E-9 FAC and the 2010 FBC 424.1   From: www.myfloridaeh.com/water/swim/index.html