CHECK OFF IF							- For assistance, call the BRC: (850) 245-4545

SUBMITTED	APPLICATION ITEM	NOTES	- BRC website: http://www.state.fl.us/health/radiation

_____   *	Division of Corporations	- A rad. materials license is a legal document, so it must be issued to a

Registration			   legal entity authorized to do business in Fla.; verify registration w/ the

the Dept. of State’s Div. of Corp (850/488-9000 or www.ccfcorp.dos.state.fl.us)

_____   1.a.	Name/Mailing Address	- List the legal name registered with the Division of Corporations; if doing

business under a different name, list both the registered name and the

business name; the license will list both names

_____      b.	Location of Use and/or	- List the street address of the facility where records and licensed materials

Storage			   (gauges) are used & stored, or description of facility location

_____   2.	License Category/Fee	- The category for fixed devices is 3L(I); an application fee ($504 as of

11/94) is required for new license applications; there is no fee for license

renewal applications; annual/reclamation fees ($845.25 as of 11/94) are

due within 60 days of license issuance and annually thereafter

_____   3.	Purpose of Application	- Check appropriate box; if submitting a renewal app., list the license no.

_____   4.	Individual Users		- List the names of all Authorized Users (AUs), including the RSO, that

will use or supervise the use of radioactive material; must have at least 2

AUs to satisfy 64E-5.1313(4)

_____   5.	Radiation Safety Officer	- List the appointed RSO (must be an AU); include a statement that written

(RSO)			   notification will be submitted to the BRC within 30 days of a change of

RSO or other management safety positions; 64E-5.212(2), .1305

_____   6.	Training and Experience	- New license applicant: Enclose copies of rad. safety training certificates

in Rad. Safety		   for RSO & each AU; training must meet Part XIII requirements; include

evidence of training in O&E procedures (often not provided by third

party training); “basic” users (limited use authorizations): 8 hrs.;

“advanced” users (authorized for gauge installations, relocations,

maintenance, surveys, etc.): 40 hrs

Renewal applicant: If AUs currently listed in license (Condition 10),state

“refer to current license”; if adding new AUs, see above;

64E-5.1305, .1307, .1310, .1313
						   all applicants: ; describe how “instructions to workers” requirement is 
						   satisfied (documentaiion is recommended; .902

_____   7.	Radioactive Material	- List the element, source model no. & maximum activity for each radio-

(RAM)			   active sealed source, & the number of sources requested

_____   8.	Use				- List the manufacturer’s name, model no. & intended use for each gauge

_____   9.	Radiation Detection		- Not required unless seeking authorization to perform advanced services

Instruments			   (gauge installations, relocations, maintenance, etc.); if used, list survey

meter’s manufacturer & model no., no. available, types of rad. detected,

min./max. range, & use; confirm access to an equivalent backup meter

when primary meter is unavailable due to calibration or repair;

64E-5.208(2), 212(2), .314

_____   10.	Calibration of Rad.		- If used, list calibration vendor's name, address & license no.; confirm

Detection Instruments	   annual calibration frequency; if requesting authorization to perform

calibrations in-house, submit cal. procedures; 64E-5.208(2), .314

_____   11.	Personnel Monitoring 	- N/A unless authorized for advanced services; if used, list type (film

(PM) Devices		   badge or TLD), exchange frequency (FB: monthly, TLD: quarterly)

& supplier (supplier list available from the BRC); 64E-5.315, .1310(1)

_____   12.	Facilities and Equipment	- Submit facility diagram showing locations of gauges, location & descrip-

tion of gauge storage area & adjacent areas, & proximity to occupied

work stations; describe security measures in place to prevent access; describe any adverse env. onditions

affecting gauges & any protective equipment or barriers installed to protect gauges or prevent exposure

from gauge rad. beams; confirm Part IX posting req. met; 64E-5.208(2), .320, .321, .901; 1315

CHECK OFF IF

SUBMITTED 	APPLICATION ITEM	  NOTES

13.	Rad. Protection Program	- Program addresses below items; 64E-5.208(2), .303

_____      A.	Member of Public (MOP)	- New lic. applicants: submit pro. for demonstrating compliance w/ MOP

Dose Study			   dose limits (< 2 mR/hr in unrestricted areas, 100 mrem/yr)

- Renewals: demonstrate compliance w/ MOP dose limits

- Contact the BRC for guidance; 64E-5.303, .313

_____      B.	ALARA Policy		- Provide description of management commitment to ALARA principles;

(As Low As			   describe RSO’s annual review of rad. protection program’s content &

Reasonably Achievable)	   implementation, & evaluation of ALARA policy’s effectiveness; a

model ALARA policy is available from the BRC; 64E-5.303

_____      C.	RSO Duties			- Describe RSO’s authority, duties & responsibilities; 64E-5.1305

_____      D.	Training Program		- Program must address instructions to workers required by 64E-5.902,

& Authorized User training required by 64E-5.1307 & .1313

_____      E.	Operating & Emergency	- O&E pro. address the items listed below; include commitment that AUs

(O&E) Procedures		   will be instructed in & have access to O&E pro; 64E-5.208(2), .1302

_____	(1)	ALARA Principles	- Procedures include instructions on proper handling & use of gauges to

to minimize occupational exposures; 64E-5.1302(1)

_____	(2)	PM Devices		- Not required unless PM devices ar used; if used, address below items

(a)	- Procedures instruct AUs working w/ gauges to always wear a PM device

(FB/TLD); 64E-5.315, .1310(1)

wear a PM device (film badge or TLD); 64E-5.315, 1310(1)

_____				(b)	- Procedures provide instructions on proper use of PM devices: only wear

assigned badge, don't expose to strong heat or light, return for exchange

when due; 64E-5.314(4), .315, .1302(4)

_____	(3)	Security		- Procedures include instructions on locking and securing stored gauges to

prevent unauthorized access; 64E-5.320, .321, .1311, .1302(3)

_____	(4)	RAM Package	- Procedures include instructions on ordering, receiving & opening RAM

Receipt Pro.		   packages (gauges); 64E-5.327, .1302(9)

_____	(5)	Lock-out		- Procedures provide instructions for when & how gauge lock-outs are

Procedures		   required; lock-out procedures posted per 64E-5.315

_____	(6)	Advanced Services	- Pro.provide instructions for gauge installations, relocations, removals,

Procedures		   surveys, etc. (if seeking approval for advanced services); 64E-5.1302

_____	(7)	Radiation Surveys	- Procedures include instructions on when radiation surveys are required

(1) whenever damage to a gauge is suspected, a survey is needed; follow

emer. pro.; RSO will arrange for survey; (2) whenever advanced services

are performed, surveys required; .1302(2), .1314

_____	(8)	Emergency		- Procedures provide instructions for gauge damage, loss, theft or accident;

Procedures		   emergency notification procedure must list RSO’s name & phone no. &

BRC 24 hr. emer. phone no. (407) 297-2095; 64E-5.1302(5) & (6)

_____	(9)	Records		- Procedures address record-keeping requirements; include a commitment

to maintain copies on file at perm. facility of manufacturers’ op./main.

manuals for each gauge model in use; 64E-5.212(2), 1302(7)

_____      F.	Leak Testing (LT)		- LT procedure describes the interval LTs are performed (1 - 3 years,

depending on the gauge model); requires LT wipes to be taken only by

AUs; lists the manufacturer’s name & model no. of the LT kit used; &

name of the vendor performing LT sample analysis; 64E-5.208, .1303

_____      G.	Inventory			- Procedure describes instructions for performance of annual physical

inventories; include a copy of a sample inventory form meeting

64E-5.1304 requirements

_____    14.	Waste Disposal		- Commit to disposing of gauges by return to the manufacturer or

transfer to another licensed recipient; 64E-5.328

_____    15.	Certificate			- Application signed & dated by a certifying official