- This checklist is for applicants using DOH Form DH 1054 to apply for or renew a category 3L(I) radioactive materials license authorizing possession and use of portable gauging devices.  DOH Regulatory Guide 1.10 provides detailed instructions for preparing the application.  Appendices, exhibits and supplements referenced below are found in the guide, which is available on the Internet at <http://www.doh.state.fl.us/health/radiation/> or by calling the FL BRC (850/245-4545).

- U.S. DOT regulations (49 CFR) are available on the Internet at <http://www.access.gpo.gov/nara/cfr/index.html>.

- To prevent the potential for identity theft, never submit documentation that lists individuals’ social security numbers or birth dates.

| CHECK WHEN ADDRESSED |  |  | ### APPLICATION ITEM | NOTES |
| --- | --- | --- | --- | --- |
|  | 1.a. | ## Name/Mailing Address |  | – Unless applying as an individual, list the business name registered with the FL Div. of Corporations; name registration may be verified by phone (850/488-9000) or online at <http://ccfcorp.dos.state.fl.us/corpweb/inquiry/corinam.html>; if doing business as (d/b/a) a fictitious name, add “d/b/a Fictictitious Name” - Under “Mailing Address,” list address to be used for license-related correspondence |
|  | 1.b. | Location of Use and/or Storage |  | - For location of use, list “temporary job sites”; for location of storage, list the street address of the permanent facility where gauges will be stored & where records will be available for inspection; if gauges will also be stored overnight at temporary job sites, add “& at temporary job sites”; 64E-5.208(2), .213(5) |
|  | 2. | License Category/Fee |  | - The license category is 3L(I); a non-refundable application fee ($605 as of 3/01) is required for new license applications; there is no fee for license renewal applications; annual/reclamation fees ($1,014.30 as of 3/01) are due within 60 days of license issuance & annually thereafter; 64E-5.204 |
|  | 3. | Purpose of Application |  | - Check the appropriate box; if applying to renew a license, list the license number |
|  | 4. | Individual Users |  | - List the name of the RSO & all authorized users (AUs) – individuals trained to use or supervise the use of portable gauges; 64E-5.208(2) |
|  | 5. | Rad. Safety Officer (RSO) |  | - List the name of the RSO (must be an AU); 64E-5.208(2), .1305(1) |
|  | 6. | Training and Experience in Radiation Safety |  | - Enclose relevant documentation on training for the RSO & each AU (i.e., training certificates, not resumes); min. of 8 hrs training covering 64E-5.1307(1) subjects; if training provided by a third party, it may be necessary to submit a separate record of training on in-house O&E procedures; a description of experience not required unless seeking approval to be an instructor for in-house training; 64E-5.208(1), .1305, .1307 |
|  | 7. | Radioactive Material |  | - List the element, source manufacturer & model no., maximum activity for each source & no. of sources requested; 64E-5.208(2) |
|  | 8. | Use |  | - List the manufacturer, model no. & intended use for each device, & brief description of the intended use(s); 64E-5.208(2) |
|  | 9. | Radiation Detection Instruments |  | - A survey meter is not typically required, but if used, list the manufacturer, model no., detection capability & range of each instrument; 64E-5.208(2) - If using a survey meter, confirm access to an equivalent backup instrument when the primary meter is unavailable due to calibration or repair; 64E-5.208(2) |
|  | 10. | Calibration of Radiation Detection Instruments |  | - If applicable, list the name, license no. & address of the instrument calibration vendor (may include option of using other licensed vendors) & confirm annual calibration frequency; if seeking to conduct in-house calibrations, submit detailed procedures (additional guidance is available from the FL BRC); 64E-5.208(2), .314 |

| CHECK WHEN ADDRESSED |  |  | ### APPLICATION ITEM | NOTES |
| --- | --- | --- | --- | --- |
|  | 11. | Personnel Monitoring (PM) Devices |  | - List the type of whole body PM badge (e.g., FB, OSLD, TLD) used, the supplier, & exchange frequency (at least quarterly); if using gauges containing Am-241 sources, PM badges must be capable of detecting neutrons; a list of approved suppliers is available from the FL BRC; 64E-5.208(2), .314., .31564E-5.208(2), .314., .315 |
|  | 12. | Facilities & Equipment |  | - Submit a diagram of the permanent facility showing the gauge storage location & all adjacent areas; indicate the storage area’s proximity to work stations (see Exhibit A for a sample facility diagram); 64E-5.208(2) - Describe the gauge storage area (e.g., cabinet, locker, closet) demonstrates adequate capacity for max. no. of gauges authorized by license; 64E-5.208(2) - Describe security at the perm. facility to prevent access to stored gauges (minimum of 2 locks between the gauge & the public); 64E-5.208(2), .320, .1311 |
|  | 13. | Rad. Protection Program |  | 64E-5. 208(2), .303 |
|  | A. | Member of Public (MOP) Dose Study |  | - Refer to Appendix A for guidance; 64E-5.208(2), .303, .313 - New license applicants: submit procedures for demonstrating compliance with MOP dose limits (< 2 mrem in any 1 hr in unrestricted areas, < 100 mrem/yr) - Renewals: submit completed study demonstrating compliance with public dose limits |
|  | B. | ALARA Policy |  | - Submit policy describing management's commitment to ALARA principles & to performance of an annual rad. protection program review/ALARA audit; App. B is a model ALARA policy; Ex. F is a model audit form; 64E-5.208(2), .303 |
|  | C. | Radiation Safety Officer |  | 64E-5.208(2), .1305 |
|  | (1) |  | #### RSO Duties | - Describe the RSO’s duties, equivalent to the duties listed in 64E-5.1305(3); App. C is a model procedure; Supplement B is model RSO administrative procedures |
|  | (2) |  | Notification of RSO Change | - Submit a statement confirming that written notification will be submitted to the BRC within 30 days of a change of RSO or other safety positions (App. C includes this commitment); 64E-5.213(7) |
|  | D. | Radiation Safety Training Program |  | - Submit a program that addresses all training components described below; App. D is a model program; 64E-5.208(2), .1307(2); .1312, .1313, .1501, .1502, 49 CFR |
|  | (1) |  | #### Instructions to Workers | - Describe how “Instructions to Workers” (radiation awareness) training will be provided to occupational radiation workers per 64E-5.902 |
|  | (2) |  | AU Training | - Describe how AU training will be provided to all gauge operators per 64E-5.1307 & .1312 [min. of 8 hrs. formal training covering .1307(1) subjects]; identify any third parties used to provide rad. safety training - If seeking to conduct AU training in-house, describe the proposed course per 64E-5.1307(2), including instructor qualifications, course description, method of testing (w/ sample exam) & retesting; Sup. D is a model training course outline |
|  | (3) |  | Hazmat Employee & Driver Training | - Describe hazmat employee & driver training provided to satisfy 49 CFR 172 & 177.816; training may be done in-house or by third parties, & may be part of rad. awareness/AU training; address record-keeping requirements per 49 CFR 172.704 |
|  | E. | PM Program |  | - Submit instructions for administration of the PM program; App. E is a model procedure; 64E-5.208(2), .315, .339, .903, .1302 |

| CHECK WHEN ADDRESSED |  |  | ### APPLICATION ITEM |  | NOTES |
| --- | --- | --- | --- | --- | --- |
|  | 13. | Rad. Protection Program |  |  | (contd.) |
|  | F. | Operating & Emergency (O&E) Procedures |  |  | App. F provides model O&E procedures; 64E-5.208(2), .1302 |
|  | (1) |  | #### Procedure Availability |  | - Refer to Sec. II of App. F; 64E-5.208(2), .1302 |
|  | (i) |  |  | O&E Procedures | - Provide instructions to have a copy of the O&E procedures accompany portable gauges at all times |
|  | (ii) |  |  | # Gauge Manuals | - Provide a commitment to maintain copies of the manufacturer’s operation/mainte-nance manuals for each gauge model authorized by the license on file for reference |
|  | (2) |  | PM Procedures |  | - Submit instructions for use of PM badges; Sec. III of App. F is a model procedure; 64E-5.303(2), .1302(1) |
|  | (3) |  | General Rules of Use |  | - Sec. IV of App. F provides model procedures; 64E-5.208(2), .303(2), .1302(1) |
|  | (i) |  |  | ALARA Principles | - Provide instructions on proper handling & use of gauges to minimize exposures |
|  | (ii) |  |  | # Precautions | - Provide instructions to: (a) use gauges in accordance w/ manufacturer’s instructions; (b) not open/remove sources from source holders; (c) prohibit contact with or direct viewing of source rod or placement of hands, etc. in radiation field |
|  | (iii) |  |  | Radiation Surveys | - Provide instructions to have the RSO arrange for a survey to be performed if damage to a gauge is suspected |
|  | (4) |  | Security |  | - Provide instructions on securing gauges to prevent unauthorized access, including a commitment to maintain direct surveillance when not in storage, & to provide two locks between a stored gauge & the public; refer to Sec. V of App. F for model procedures & Ex. C for a model utilization log (recommended for users of multiple gauges); 64E-5.320, .321, .1302(3); .1311 |
|  | (5) |  | Transportation |  | - Provide instructions for safe transport of gauges per U.S. DOT regs (inspection, packaging, marking, labeling, blocking & bracing, security, shipping papers, emergency response information (ERI), etc.; Sec. VI of App. F is a model procedure; Ex. D-1 & Ex. D-2 are model shipping papers; Ex. E is a model ERI sheet; Sup. C provides guidance on SI units; 64E-5.321, .1311, .1302(5), .1501, .1502; 49 CFR |
|  | (6) |  | Routine Maintenance |  | - Provide instructions to: (a) restrict performance of routine gauge maintenance & cleaning to AUs in accordance w/ manufacturer’s instructions; & (b) prohibit non-routine gauge maintenance or repair requiring removal of the source/source rod; Sec. VII of App. F is a model procedure; 64E-5.208(2) |
|  | (7) |  | Posting |  | - Provide instructions to address posting requirements specified in 64E-5 Parts III, IX & XIII; Sec. VIII of App. F is a model procedure; 64E-5.208(2), .323, .901, .1302 |
|  | (8) |  | Ordering, Receiving, Opening & Shipping |  | - Provide instructions for preparation & handling of incoming & outgoing gauge shipments; App. H is a model procedure; 64E-5.208(2), .327, .1501, .1502, 49 CFR |
|  | (9) |  | Special Procedures for Depth Probes |  | - If requesting authorization for depth probes (gauges capable of extending to depths > 3'), include O&E procedures providing instructions specific to those gauges (e.g., CPN Model 501, 503, Troxler Model 4300 Series); 64E-5.208(8), .212(2), .1302 |

| CHECK WHEN ADDRESSED |  |  | ### APPLICATION ITEM | NOTES |
| --- | --- | --- | --- | --- |
|  | 13. | Rad. Protection Program |  | (contd.) |
|  | G. | Emergency Procedures |  | - Provide instructions for handling gauge loss, theft or damage; include emergency notification numbers for the RSO & FL BRC; App. G is a model procedure; 64E-5.208(2), .1302, 49 CFR 173.1435 |
|  | H. | Leak Testing (LT) |  | - App. I is a model procedure; 64E-5.208(2), .1303 |
|  | (1) |  | #### LT Kit & Vendor | - List: (a) manufacturer name & model no. of LT kit used, & (b) name & license no. of vendor contracted to perform LT sample analysis (may include option of using other licensed vendors) |
|  | (2) |  | #### LT Procedure | - Provide instructions for: (a) the interval LTs will be performed (6/12 months) & (b) collecting LT samples |
|  | I. | Inventory |  | - Submit procedure for performance of semiannual physical inventories, including sample inventory form; App. J provides a model procedure; Ex. B is a model inventory form; 64E-5.208(2), .1304 |
|  | J. | Record Retention Procedure |  | - Submit procedure addressing record-keeping requirements specified in 64E-5; App. L is a model procedure; 64E-5.208(2), .1302 - Provide commitment to maintain on file, for at least 1 year after the last shipment, copies of the testing methods & results for each Type A package in use (contact gauge manufacturers for the documentation); 64E-5.1502(2), 49 CFR 173.415(a) |
|  | 14. | Waste Disposal |  | - Submit procedure with a commitment that gauges will be transferred only to licensed recipients (e.g., manufacturer, waste broker); App. K is a model procedure; 64E-5.208(2), Part III, .1302 |
|  | 15. | Certificate |  | - Have application signed & dated by a certifying official – a person authorized to make legally binding statements on behalf of the applicant; e.g., president, CEO, etc. |

Notes: