I.	Introduction to SI Units

SI (Systeme International) units comprise the primary measurement system for most countries.  The system is also finding increasing use in the United States.  State and federal regulatory agencies, including the Florida Bureau of Radiation Control and the U.S. Nuclear Regulatory Commission, have adopted SI units for radiation measurements; other agencies (e.g., the U.S. Department of Transportation) require their use.

II.	Common Radiological Unit Prefixes

| Submultiples |  |  |  | Multiples |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | milli | 10-3 | thousandth | k | kilo | 103 | thousand |
|  | micro | 10-6 | millionth | M | mega | 106 | million |
| n | nano | 10-9 | thousand millionth | G | giga | 109 | thousand million |
| p | pico | 10-12 | million millionth | T | tera | 1012 | million million |

III.	Length

| 1 centimeter (cm) | = | 0.3937 in | = | .03287 ft |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 meter (m) | = | 100 cm | = | 39.37 in | = | 3.281 ft |
| 1 inch (in) | = | 2.54 cm | = | 0.0254 m |  |  |
| 1 foot (ft) | = | 30.48 cm | = | 0.3048 m |  |  |

IV.	Activity

The traditional unit is the Curie (Ci); the SI unit is the Becquerel (Bq)

1 Ci = 3.7 x 1010 Bq = 37 GBq 1 Bq = 1 disintegration per second = 2.7027 x 10-11 Ci or  27 pCi

To convert Bq to Ci, divide the Bq figure by 37 x 109 (or multiply the Bq figure by 2.7027 x 10-11)

To convert Ci to Bq, multiply the Ci figure by 37 x 109

| OLD | 1 pCi |  | 27 pCi |  | 1 nCi |  | 27 nCi |  | 1 Ci |  | 27 Ci |  | 1 mCi |  | 27 mCi |  | 1 Ci |  | 27 Ci |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Curie) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NEW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (becquerel) | 37 mBq |  | 1 Bq |  | 37 Bq |  | 1 kBq |  | 37 kBq |  | 1 MBq |  | 37 MBq |  | 1 GBq |  | 37 GBq |  | 1 TBq |  |

Examples:		9 mCi = 333 MBq = 0.333 GBq			10 mCi = 370 MBq = 0.37 GBq
			44 mCi = 1628 MBq = 1.63 GBq		50 mCi = 1850 MBq = 1.85 GBq

IV.	Activity	  (continued)

# Table A					     Table B

| Curie Units | Becquerel Units |  |  | Curie Units | Becquerel Units |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ci | kBq |  |  | Ci | MBq |  |
| mCi | MBq |  |  | mCi | GBq |  |
| Ci | GBq |  |  | Ci | TBq |  |
| 0.1 | 3.7 |  |  | 50 | 1.85 |  |
| 0.25 | 9.25 |  |  | 60 | 2.22 |  |
| 0.5 | 18.5 |  |  | 100 | 3.7 |  |
| 0.75 | 27.75 |  |  | 200 | 7.4 |  |
| 1 | 37 |  |  | 250 | 9.25 |  |
| 2 | 74 |  |  | 500 | 18.5 |  |
| 3 | 111 |  |  | 800 | 29.6 |  |
| 5 | 185 |  |  | 1000 | 37 |  |
| 7 10 20 25 | 259 370 740 925. | From Table B: 50 mCi = 1.85 GBq 3.7 MBq = 100 Ci To convert from one unit to another, read across from one column to the |  |  |  |  |
| From Table A: 0.1 mCi = 3.7 MBq 0.1 Ci = 3.7 GBq |  |  | other, ensuring the units are in the same line of the column headings. |  |  |  |

V.	Radiation Dose Equivalent

| The traditional unit is the rem; the SI unit is the sievert (Sv). |  |  |
| --- | --- | --- |
| 1 rem = 0.01 sievert (Sv) = 10 mSv | 100 rem = 1 Sv | 500 rem = 5 Sv |
| 1 rad = 0.01 gray (Gy) = 10 mGy | 100 rads = 1 Gy | 500 rads = 5 Gy |
| The working SI unit is the sievert (Sv) |  |  |

| OLD | 0.1 mrem |  | 0.5 mrem |  |  | 1.0 mrem |  |  | 5 mrem |  | 10 mrem |  | 100 mrem |  |  | 500 mrem |  |  | 1 rem |  | 5 rem |  | 10 rem |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (rem) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NEW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (sievert) | 1 Sv |  |  | 5 Sv |  | 10 Sv |  | 50 Sv |  |  | 0.1 mSv |  | 1 mSv |  | 5 mSv |  |  | 10 mSv |  |  | 50 mSv |  | 100 mSv |  |

IV.	Surface Activity

| Ci/cm2 | 10-6 |  | 3 x 10-6 |  | 10-5 |  | 3 x 10-5 |  | 10-4 |  | 3 x 10-4 |  | 10-3 |  | 3 x 10-3 |  | 10-2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bq/cm2 | 0.037 |  | 0.1 |  | 0.37 |  | 1 |  | 3.7 |  | 10 |  | 37 |  | 100 |  | 370 |  |
| (kBq/cm2) | 0.37 |  | 0.1 |  | 3.7 |  | 10 |  | 37 |  | 100 |  | 370 |  | 1000 |  | 3700 |  |