I.	Instructions for Using PM Badges

A.	General Instructions

Whole body personnel monitoring (PM) badges will be assigned by the Radiation Safety Officer (RSO) to workers when the RSO determines use of badges is appropriate.

If assigned, PM badges are to be worn during any activities where occupational radiation exposures may be received.  Badges are worn on the front of the torso, at or above the waist and below the shoulder.  Badges must be returned to the RSO at the end of each monitoring period to ensure rapid processing.  The RSO must be notified immediately if a PM badge is lost or damaged.

PM badges are individually assigned and cannot be shared.  If a spare badge is used, it must be marked with the name, initials and/or identification number of the individual designated to wear it.  Badges cannot be worn during non-occupational radiation exposures (e.g., medical or dental x-rays, etc.).

If a badge is lost or damaged, an estimate of the worker’s dose for the period covered by the badge must be provided to the badge vendor and kept on file.  If a spare badge is used for the remainder of the monitoring period, the dose recorded on the spare badge must be added to the estimated dose to obtain the worker’s total occupational dose for the monitoring period.

- Recommended Work Practices for Personnel Monitoring - Never leave PM badges in close proximity to a source of radiation. - Protect badges from moisture, chemicals, intense heat or light. - When not in use, store badges with the control badge in a low background radiation area.

B.	Special Instructions for New Hires and Lost/Damaged Badges

A spare badge may be assigned to a new employee until the badge vendor can be notified to issue a badge bearing the worker’s name for the next monitoring period.  Spare badges may also be used to replace a badge that has been lost or damaged before the end of the monitoring period.  To ensure their use by only one individual, spare badges will be imprinted with the worker’s name or another form of identification.  Workers using spare badges will have the dose recorded by the badge added to their occupational dose record.  In the event of a lost/damaged badge, the RSO will estimate the worker’s dose for the period the badge was worn, and notify the badge processor if the individual’s dose record needs to be revised.

II.	PM Record Requirements

A.	Records of Prior Occupational Dose

Every reasonable effort will be made to obtain the lifetime cumulative occupational radiation dose from each monitored worker.  If an individual is unable to provide their lifetime cumulative exposure records, records will be obtained from their previous employer.  The records must include all of the information required by section 64E-5.308, F.A.C., using Form DH-1623 or an equivalent form.

II.	PM Record Requirements    (contd.)

B.	Records of Personnel Monitoring Results

Records of doses received by each monitored worker will be reviewed by the RSO within 30 days of receipt to determine if unnecessary exposures are being received.  The RSO will sign and date each report reviewed and will investigate within 30 days the cause of any personnel exposure considered to be excessive.  If warranted, the RSO will take corrective actions to prevent recurrence.  A report of each investigation and the actions taken, if any, will be documented and maintained for inspection purposes.

PM records will be maintained as long as the license remains in effect.  The records will be kept on Form DH-1622 or an equivalent form and will contain all of the information required by section 64E-5.339, F.A.C.  These records will be updated annually.

C.	Annual Reports to Monitored Individuals

Each worker assigned a PM badge will receive a written annual exposure report describing the past year’s monitoring results, as required by section 64E-5.903, F.A.C.  Records documenting that the reports have been furnished to monitored workers will be maintained for 3 years.

D.	Termination Reports to Monitored Individuals

Within 30 days of termination of employment, or within 30 days after the individual’s exposure has been determined, whichever is later, each monitored worker will receive a written exposure report summarizing the individual’s occupational radiation exposure, as required by section 64E-5.903, F.A.C.  Records showing reports have been furnished to monitored workers will be retained for at least 3 years.

E.	Records for Female Workers and Declared Pregnancies

Upon hiring, female personnel assigned to work with gauges will be provided verbal instructions concerning the potential risks involved for pregnant women exposed to radiation and a copy of U.S. NRC Regulatory Guide 8.13 (“Instruction Concerning Prenatal Radiation Exposure” – Rev. 3, 6/99).  Following receipt of the instructions and guidance, female workers will sign an Instructions for Women Working With Radiation form (Exhibit F or equivalent) to document receipt of the instructions.

Pregnant women will be provided verbal instructions to always wear their assigned PM badge at waist level to estimate the embryo/fetus dose.  Such workers will sign an Instructions for Declared Pregnant Women form (Exhibit G or equivalent) to document receipt of instructions on PM requirements during pregnancies and a Declaration of Pregnancy form (Exhibit H or equivalent) that includes the estimated date of conception.  The forms will be retained until license termination.

Fetal doses will be kept ALARA, and will not be allowed to exceed 500 millirem during the entire pregnancy as a result of occupational exposures.  The Bureau of Radiation Control (BRC) recommends that an embryo/fetus not receive more than 50 millirem in any one month.  Records of fetal dose will be documented in an Occupational Radiation Dose Record (Form DH-1623 or equivalent) and the forms, with the dose records of the declared pregnant woman, will be retained until license termination.

II.	PM Record Requirements	(contd.)

F.	Occupational Dose Limits for Minors

Minors are not allowed to receive an annual occupational dose exceeding 500 millirem.  Dose records to minors will be documented with the Cumulative Occupational Radiation Dose Record (Form DH-1623 or equivalent) and Occupational Radiation Dose Record (Form DH-1622 or equivalent) as appropriate, and the records will be retained until license termination.

G.	Worker Overexposure Reports

When a report of an individual’s exposure must be sent to the BRC as required by section 64E-5.347, F.A.C., the exposed individual will also be notified no later than when the report is sent out.