I.	Radiation Warning Signs	64E-5.322, .323 & .324, F.A.C.

When required, areas or rooms where fixed gauges are located will be posted with a conspicuous sign or signs bearing the radiation symbol and the words “Caution (or Danger), Radioactive Material(s).”  The criteria for posting an area with a “Caution (or Danger), Radioactive Material(s)” sign is specified in subsection 64E-5.323(5), F.A.C.

Accessible areas where radiation levels exist that could result in an exposure greater than 5 millirem in any 1 hour at 30 cm from the source of radiation, or from any surface that the radiation penetrates must be posted with a conspicuous sign or signs bearing the radiation symbol and the words “Caution, Radiation Area.”

II.	Emergency Procedures	64E-5.901, F.A.C.

Emergency procedures approved by the Bureau of Radiation Control (BRC) must be conspicuously posted for quick reference in the event of an emergency.  The BRC radiological notification poster may also be posted.

III.	Lockout Procedures	64E-5.1315, F.A.C.

BRC approved procedures describing instructions for locking out fixed gauges will be conspicuously posted for quick reference in order to prevent workers from entering a gauge’s radiation beam during work in, on or around installed gauges.

IV.	“Notice to Employees” Form	64E-5.901, F.A.C.

A current BRC “Notice to Employees” form must be conspicuously posted for review by workers.  The notice describes radiation control regulations, employer and worker responsibilities, radiation exposure reporting requirements, and inspection requirements.

V.	Other Required Documents	64E-5.901, F.A.C.

The documents listed will be conspicuously posted, unless the “Notice to Employees” form is used to identify where they can be examined by workers (the form has space available to list where the other documents may be examined).

- Parts III and IX of Chapter 64E-5, Florida Administrative Code

- Radioactive materials license

- Operating procedures

VI.	Notices of Violations and Related Documents	64E-5.901, F.A.C.

Notices of violations, proposed imposition of administrative penalties, or BRC issued orders will be posted within 5 working days after receipt.  Responses to such BRC documents will be posted within 5 working days after dispatch.  These documents will remain posted for a minimum of 5 working days or until action correcting the violation(s) has been completed, whichever is later.