I.	In the event of a stolen, lost or missing gauge, immediately notify the Radiation Safety Officer (RSO) and the Bureau of Radiation Control (BRC).

II.	Implement the following procedure in the event of physical damage to a gauge or any other emergency or unusual situation:

A.	Immediately secure the area around the gauge and keep people at least 30 feet away until the situation is assessed and radiation levels are known.  Maintain surveillance of the perimeter to prevent unauthorized entries.

B	Care for life-threatening injuries first, even if individuals may be contaminated.  Perform first aid and remove them from the area only when medically safe to do so.  Evaluate the situation to determine if anyone may have been exposed to radiation.  Notify emergency personnel and hospital staff about possible radioactive material contamination.  Do not allow any potentially contaminated people to leave the scene; have them remain at least 30 feet from the damaged gauge until they can be surveyed for contamination.

C.	If any equipment is involved, isolate the equipment until it can be surveyed for possible contamination.

D.	As soon as possible, notify the RSO and the BRC using the numbers listed below.  Follow the directions provided by the RSO and the BRC.  Refer to notification and reporting procedures for additional instructions.

E.	Wait for technical assistance prior to approaching or moving the gauge or any other involved equipment until the extent of contamination has been determined.  A survey meter must be used to determine the presence of contamination in the area or on personnel.  Special precautions and protective clothing/equipment must be used to perform decontamination and disposal of any contaminated materials.

F.	Arrange for a radiation survey to be conducted as soon as possible by a qualified person using appropriate radiation detection instrumentation.  To accurately assess the radiation danger, it is essential that the person performing the survey be competent in the use of the survey meter.

G.	If radiation levels permit it, perform a leak test on the source holder to determine if the sealed source is leaking.  If a leak test kit is unavailable, use a Q-tip (with one end cut off), to perform the test, and place the Q-tip in a zip-lock bag before sending it off for expedited analysis.

Radiation Safety Officer: _____________________________________ RSO Phone No.: (W):_________________ (H):____________________ Florida Bureau of Radiation Control 24-Hour Radiological Emergency Notification No.: (407) 297-2095