POTENTIAL HAZARDS

###### IMMEDIATE HAZARDS TO HEALTH

- External radiation hazard from unshielded radioactive material.

- Low-level radioactive material; little personal radiation hazard when shielded.

- Materials in special form are not expected to cause contamination in accidents.

- Some radioactive materials cannot be detected by commonly available instruments.

- Potential internal radiation hazard from inhalation, ingestion, or breaks in skin, only if special form capsule is breached.

#### FIRE OR EXPLOSION

- No risk of fire or explosion.

- Radioactivity does not change flammability or other properties of the materials.

EMERGENCY PROCEDURES

###### IMMEDIATE PRECAUTIONS

- Isolate hazard area to within a 10-15 foot radius of the gauge and restrict access.

- Emergency response actions may be performed prior to any measurement of radiation; limit entry to shortest time possible.

- Notify local authorities and Radiation Control Authority of accident conditions.

- Detain uninjured persons, isolate equipment with suspected contamination, and delay cleanup until receiving instruction from Radiation Control Authority.

#### FIRE

- Do not move damaged containers; move undamaged containers out of fire zone.

- Small Fires:  Dry chemical, CO2, water spray, or regular foam.

- Large Fires:  Water spray, fog (flooding amounts).

#### SPILL OR LEAK

- Do not touch damaged containers or exposed contents.

- Damage to outer container may not affect primary inner container.

- Special form capsules are not expected to leak as a result of an accident or fire.

#### FIRST AID

- Use first aid treatment according to the nature of the injury.

- Advise medical personnel that victim may be contaminated with low-level radioactive material.

- Except for the injured, detain persons exposed to radioactive material until arrival or instruction of Radiation Control Authority.