Semiannual inventories are required to account for the sealed sources used as calibration and references sources or contained in a device that is possessed under a radioactive materials license.  To ensure accountability of radioactive material, the procedure described below will be followed.

I.	Physical Inspection

At least every six months, evaluate the general condition of each sealed source and/or device to determine if any damage has occurred.  In addition, verify that all of the identification and warning labels remain attached and are legible.

If the inspection reveals missing labels, evidence of damage, or adverse environmental conditions immediately report the problem(s) to the Radiation Safety Officer (RSO).  If warranted, arrange to have the sealed source and/or device’s radiation levels measured.  If excessive radiation levels are discovered, notify the Bureau of Radiation Control at 
(407) 297-2095.

II.	Inventory Records

Retain inventory records for three years from the date of the inventory.  The attached inventory form (or equivalent) must be used.  Required inventory information includes:

- Device manufacturer, model number and serial number

- Source manufacturer, model number and serial number

- Source identity and estimated activity

- Location

- Condition

- Date of inventory

- Signature of the Radiation Safety Officer (or the RSO’s designee)