PLANS REVIEW CHECK LIST – WADING POOL - DOH/BOAF version May 3,2012

Date_____________

County_____________________________________ Date Received__________________ Log#_____________

Project_____________________________________________________________________________________

Engineer_______________________________________________ Original____ Revision____ Modification____

Reviewed By_____________________________________ SP-_______________________File #____________

Date Re-Submittal Received:___________________________ Date of 2nd Review:_______________________

_____________________________________________________________________________________________

Items needing correction or clarification are marked by an "X" beside the appropriate section number of the Florida Administrative Code citation (2009 64E-9, FAC, or 2010 FBC 424.1).  We have left the 64E-9 requirements in this checklist because they are critical for public health and therefore the pool will be checked for these items by the County Health Department at the first operating permit inspection after the Building Official’s approval of the construction.

Reviewer:  Note that section 424.1.7.1 Requires that all “Wading Pools shall meet the requirements of Sections 424.1.1 through 424.1.6.5, unless otherwise indicated.

Use the Pool Checklist for the sections 424.1.1 (Flood Hazard Areas) through 424.1.6.5 (Recirculation and Treatment Systems) in addition to this checklist for IWFs.

Florida Building Code

Section

424.1.7.1	____ Wading pool is not physically connected to any other pools.

424.1.7.1	____ Wading pool is in compliance with all other applicable sections of the FBC.

424.1.7.2	____ Wading pool depth does not exceed two feet.

424.1.7.2	____ Wading pool depth at the pool perimeter is uniform, except for pools with zero depth

entry portion, and does not exceed 12 inches.  The pool  floor shall not be more than

12 inches below the deck unless steps and hand-rails are provided.

424.1.7.3	____ Wading pool recirculation/filtration rate provides for at least one turnover every hour.


424.1.7.3	____ Wading pool main drain lines discharge into a collector tank.


424.1.7.4	____ If wading pool has a perimeter of 20 feet or less, at least two equally spaced adjustable

return inlets are provided.


424.1.7.5	____ Wading pool is provided with a drainage to waste line (without cross connection) with 
		a quick opening valve.


424.1.7.6	____ Wading pool has provision for vacuuming, if pool water surface area is 200 square feet 
	or greater. There is no provision for direct suction where vacuum port is in pool floor or wall.


424.1.7.7	____	If wading pool is adjacent to other pool within 50’, it is enclosed by a min. 48” high

barrier or fence with self closing and latching gates.  (within 40’ if adjacent to area <1’

deep of zero-depth entry pool)

424.1.7.7	____ A minimum 10' wide deck is provided around at least 50% of the wading pool perimeter 
		with the remainder of the perimeter deck being at least 4' wide.

424.1.7.7	____ At least 10 feet of deck exists between adjacent swimming pool and wading pool.

424.1.7.3.1	____ Skimmer equalizer lines (when required) are plumbed into the main drain.

424.1.7.3.2	____ The grate cover shall be sized such that the flow velocity does not 
		exceed 1 1/2 feet per second when the equalizer line is operating.

424.1.7.8	____ Overhead lighting is required for night use in accordance with 424.1.4.2.1

6E-9.009(9)	____ Automated ORP & pH controllers with sensing probes are provided.

Revised for 2009 64E-9 FAC and the 2010 FBC 424.1   From: www.myfloridaeh.com/water/swim/index.html